[image: image7.jpg]MAKiNA LIKIT GAZ IMALAT iN,§.
SANAYii ve TICARET LTD.STI



[image: image8.jpg]MAKiNA LIKIT GAZ IMALAT iN,§.
SANAYii ve TICARET LTD.STI








	APPLICATION

LC MA series flow meters are mechanical volumetric devices. They provide repetitive and precise volume measurements, regardless unstable working conditions resulting from changes of temperature and pressure of product handled. LC MA series flow meters are manufactured in wide range of sizes and in different material specifications. MA series meters are used at bottling plants, LPG terminals and in many other industrial applications.
Features:

- High precision and repeatability of measurement
- High durability, no direct contact between rotors and  internal surface of measuring chamber

- Low pressure drops
- Operation without the need of electrical power or signal conditioning (for units equipped with mechanical registers, printers or presets)
- Can be equipped with electronic registers
PARAMETERS

Flow:                             50 – 1325 l/min

Working pressure:        25 - 21,0 bar

Working temperature:  -40o do +70o C

Flanges:             M-Series: NPT,BSP or weld flanges

                          MS-Series: ANSI and DIN flanges
AVAILABLE ACCESSORIES

- Mechanical counter with totalizer
- Printer

- Strainer, air eliminator, pressure control valves, differential valve
- Adapters

- Check valves

- Thermometers

- Presets
	[image: image1.jpg]



[image: image2.jpg]= fle

@_WT .




[image: image3.jpg]





[image: image4.jpg]



Semi-trailer skid installation with MA7 flow-meter 

TECHNICAL DATA

	Cat No.
	Maximum flow
	Size
	Connection
	Material
	Maximum working pressure

	Nr
	
	
	
	
	10,5 bar
	25 bar

	17.001
	113 l/min
	DN 40
	1 ½”
	Aluminum
	
	MA-4

	17.002
	227 l/min
	DN 40

DN 50
	1 ½”

2” 
	Aluminum

Stainless steel
	M-5

M-5
	MA-5

	17.003
	380 l/min
	DN 50

DN 40
	2”

1 ½”
	Aluminum

Stainless steel
	M-7

M-7
	MA-7

	17.004
	757 l/min
	DN 80
	3”
	Aluminum
	MS-15
	MA-15

	17.005
	1325 l/min
	DN 80
	3”
	Aluminum
	M-30
	MA-30


SIZE

[image: image5.png]Front view Top view

Mounting plate

mensions: flow meters with electronic registra
O G Note: Dimensions shown are not for construction use.





	Mod
	Size

	
	
	A
	B
	C
	D
	E
	F
	G
	H
	I
	J
	K
	L
	M
	N
	O

	M-5
	mm
	360
	135
	105
	172
	14
	48
	125
	213
	219
	352
	24
	119
	233
	81
	116

	M-7
	mm
	362
	137
	105
	171
	14
	71
	106
	213
	241
	378
	24
	119
	259
	106
	141

	M-15
	mm
	360
	135
	105
	172
	14
	48
	125
	213
	273
	411
	32
	127
	343
	129
	167

	M-30
	mm
	521
	270
	143
	230
	22
	122
	200
	n/a
	351
	494
	36
	181
	313
	167
	195


STANDARD ACCESSORIES CONFIGURATIONS
[image: image6.png][
VBN

]
ZENIE

Meter with counter Meter with counter, Meter with counter, preset Meter with counter, preset
strainer and air eleminator counter and preset valve counter, preset valve,
strainer and air eliminator





Liquid Controls Flow Meters








�




















�








Liquid Controls Flow Meters













